# 8D Pre-Submission Checklist — 24 reasons reports come back

8D / SCAR number: _______________________________

Customer / part: _______________________________

Submission date: _______________________________

Customer reviewer (SQE): _______________________________

Checked before sending by: _______________________________

Work down the list before you submit. Anything marked Fix is a rejection waiting to happen; anything marked Pass needs a location in the last column, because a reviewer will ask for it.

| # | D | What the reviewer checks | Rejected when | Accepted when | Status | Evidence — where it is |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | D0 | The symptom is the customer's own wording | The complaint has been reworded into internal language, or a part number replaces the description. | The complaint text, the customer reference and the date it was received appear as they arrived. |  |  |
| 2 | D0 | Emergency response is recorded | The first action taken on day one is missing, or has no date and no owner. | The immediate action, the date and time, and who took it are all recorded. |  |  |
| 3 | D1 | The team is cross-functional and real | The list is managers only, with nobody from the process, or a single name carries the whole report. | Three to seven members are named with roles, including the process owner and a champion with authority to commit resources. |  |  |
| 4 | D2 | The problem is quantified | The section restates the complaint in prose: 'customer found defective parts'. | What, where, when and how many are given as numbers, measured against the target or specification. |  |  |
| 5 | D2 | The boundary is defined | Nothing is excluded, so every candidate cause stays equally plausible. | An Is / Is Not grid (or equivalent) shows what was equally exposed and stayed unaffected. |  |  |
| 6 | D2 | The suspect population is bounded | Quantity affected, lot or date codes and the traceability limits are missing. | The population is bounded by lot, date code or serial range, and the bound is justified. |  |  |
| 7 | D3 | Containment actually contains | The action is 'operators informed' or 'told to pay attention' — an instruction, not a barrier. | A physical or procedural barrier is in place that the customer can verify independently. |  |  |
| 8 | D3 | Containment covers the whole pipeline | Only the plant is addressed; stock already at the customer or in transit is not mentioned. | Customer stock, goods in transit, warehouse, WIP and finished goods are each listed with quantity and disposition. |  |  |
| 9 | D3 | Containment effectiveness is measured | No effectiveness figure, no sample size, no verification data. | Check method, sample size, result and start date are stated — and the removal date is left open. |  |  |
| 10 | D4 | The occurrence cause is verified, not asserted | The chain stops at a process step or a symptom, and nothing was tested. | The cause is proven — by switching the effect on and off, or by data that excludes the alternatives. |  |  |
| 11 | D4 | The escape cause is present ← most common rejection | Only the occurrence cause is given, or the escape cause reads 'the inspector missed it'. | The control that should have detected the defect is named, with the reason it did not — a second chain with its own evidence. |  |  |
| 12 | D4 | The cause is systemic, not personal | The chain terminates at a person: 'operator error', 'human error'. | The chain reaches the system that allowed the error: method, machine capability, specification, planning or workload. |  |  |
| 13 | D4 | Both chains have attached evidence | The analysis refers to tests, measurements or data that are not in the file. | Each chain names its evidence with dates and revision numbers, and the evidence is attached. |  |  |
| 14 | D5 | One permanent action per root cause | The action list does not map to the causes, or a cause has no action against it. | Every root cause has at least one action, each with a named owner and a date. |  |  |
| 15 | D5 | The action is stronger than training | Retraining, awareness or 'increase inspection' is offered as the permanent corrective action. | The action eliminates, substitutes, error-proofs or engineering-controls. Training is listed as supporting, not as the fix. |  |  |
| 16 | D5 | The verification method is chosen before implementation | The report says the action 'will be monitored', with no acceptance criterion. | The measure, the target, the period and the person who confirms it are written before the data is collected. |  |  |
| 17 | D6 | Effectiveness is shown with data | The section says 'implemented' and gives dates only. | Before and after data over a stated period show the defect rate at target, with the sample size. |  |  |
| 18 | D6 | Containment is removed only after proof | Containment ends on the implementation date, before any effectiveness data exists. | The removal date falls after the monitoring period, and the decision to remove is recorded with the data behind it. |  |  |
| 19 | D7 | PFMEA and control plan are updated | D7 is empty, or repeats the D5 actions in different words. | Named documents with revision numbers and dates: PFMEA, control plan, work instruction, inspection standard. |  |  |
| 20 | D7 | Read-across is done | Similar parts, lines, shifts and plants are not mentioned at all. | The scope checked is named — including the conclusion that nothing else is affected, if that is the finding. |  |  |
| 21 | D8 | Closure is signed and the lesson is recorded | The report is closed without champion sign-off, or the lessons-learned field repeats the root cause. | Sign-off, date and a lesson written so that a team on another product could use it. |  |  |
| 22 | Form | The customer's own form and mandatory fields | Your layout is used where the customer requires theirs, or mandatory portal fields are left blank. | Every mandatory field is completed, or explicitly marked N/A with a one-line reason. |  |  |
| 23 | Form | Dates are internally consistent | D3 predates D0, or containment removal predates the D6 data, or the portal dates disagree with the report. | The timeline reads in order, and the dates in the report match the dates in the customer portal. |  |  |
| 24 | Form | Attachments are legible, named and referenced | Photographs with no date, no scale and no caption; files called IMG_0423; nothing pointing to which discipline they support. | Attachments are numbered, captioned, dated and referenced from the discipline that relies on them. |  |  |

## Wording that gets a report sent back

| Wording in your report | What the reviewer concludes | Write this instead |
| --- | --- | --- |
| Operator error / human error | The analysis stopped at a person, so the system that allowed it is untouched and the defect can recur tomorrow. | What made the error possible and what let it through: no error-proofing at the step, no downstream detection. |
| Operator re-trained | The weakest action in the hierarchy, and it depends on memory. Nothing in the process changed. | A change to the process or control that works without recall. Training stays, as a supporting action. |
| 100% inspection (as the permanent action) | Inspection is containment. Offering it as the permanent fix says no cause was removed. | Keep the inspection as containment, name the permanent action separately, and remove the inspection with data in D6. |
| No problem found / could not reproduce | The investigation stopped without a boundary, and the customer still holds the defective part. | What was tested, with what method and sample size, and what those results exclude — plus the next test. |
| Will be monitored | No acceptance criterion, so there is no way for anyone to say later whether it worked. | The measure, the target, the period and the name of the person who confirms it. |
| Operators instructed to pay attention | An instruction is not a containment; the customer is not protected. | The barrier actually in place: sortation with results, a fixture change, a gauge, or a hold on the suspect lots. |
| Process improved / procedure updated | Nothing verifiable. There is no document to audit against. | The document name, its revision number and the date it was released. |
| Supplier issue | Responsibility has moved, but your escape cause is still missing — the part reached the customer through your controls. | Your own escape cause: why incoming inspection or the supplier control plan passed it, and what changed in your control. |
| As per standard procedure | If the defect happened while the procedure was followed, the procedure is part of the cause. | What in the procedure permitted the outcome, and what was changed in it. |

These phrases are the ones most often flagged in customer reviews. They are not forbidden words — each is rejected because of what it leaves unanswered, so the replacement column is the point, not the wording.

Free 8D pre-submission checklist — 5xwhys.com/articles/8d/rejected-by-customer/  ·  A line is passed when the evidence exists, not when the box is ticked