# OEE Shift Sheet

Date: _______________________________

Shift: _______________________________

Line / machine: _______________________________

Recorded by: _______________________________

## Time

| Item | Minutes | Note |
| --- | --- | --- |
| Shift length |  |  |
| Breaks and no-demand time |  | excluded from OEE |
| Planned production time |  | = shift − breaks |
| Stops: breakdowns |  |  |
| Stops: changeovers and setup |  | an availability loss, not a break |
| Run time |  | = planned − all stops |

## Output

| Item | Value | Note |
| --- | --- | --- |
| Ideal cycle time (s / part) |  | what the process can sustain, not nameplate |
| Total count |  |  |
| Good count |  | right first time — rework is not good |

## Result

| Factor | Formula | Value |
| --- | --- | --- |
| Availability | Run time ÷ planned production time |  |
| Performance | (Ideal cycle time × total count) ÷ run time |  |
| Quality | Good count ÷ total count |  |
| OEE | Availability × performance × quality |  |
| OEE, one-line check | (Good count × ideal cycle time) ÷ planned production time |  |

## Loss log — record the reason, not just the minutes

| Loss | Factor | Minutes / parts | What happened | Cause |
| --- | --- | --- | --- | --- |

## The six big losses

| Loss | OEE factor | Typical examples | What to record |
| --- | --- | --- | --- |
| Equipment failure | Availability | Breakdowns, unplanned maintenance, tooling failure | Minutes stopped, and the stop reason |
| Setup and adjustments | Availability | Changeovers, warm-up, first-off approval, material shortage | Minutes from last good part to first good part |
| Idling and minor stops | Performance | Jams, blocked or starved sensors, short stops cleared by the operator | Count of stops under your threshold, e.g. 5 minutes |
| Reduced speed | Performance | Running below the ideal cycle time: worn tooling, cautious settings, material | Actual cycle time against ideal |
| Process defects | Quality | Scrap and rework during stable production | Parts, with the defect type |
| Startup rejects | Quality | Scrap and rework after a changeover or restart, until the process is stable | Parts, split from process defects |

Totals are calculated from summed minutes and counts, never by averaging the shift percentages — an average gives a short bad shift the same weight as a long good one. Rework counts as a quality loss, not as good output. Use the ideal cycle time the process can sustain, not the nameplate speed.

Free OEE template — 5xwhys.com/articles/rca/oee/  ·  OEE says where the time went; the loss log says why