# PPAP Checklist — all 18 elements

Part number / revision: _______________________________

Customer: _______________________________

Submission level: _______________________________

Reason for submission: _______________________________

Prepared by: _______________________________

| # | Element | What it must contain | Where it usually fails | Status | Owner | Evidence — where it is |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Design records | The drawing and specifications at the revision on the purchase order, plus the bill of materials for assemblies. | The revision in the file is not the revision on the purchase order. |  |  |  |
| 2 | Authorized engineering change documents | Every change already in the product but not yet on the drawing, with the customer's authorization. | A change is running in production and exists only in an email. |  |  |  |
| 3 | Customer engineering approval | Evidence of approval where the customer requires it — an engineering sign-off or an approved deviation. | Assumed not required, without asking. |  |  |  |
| 4 | Design FMEA | The DFMEA for the part, if you are design-responsible. | Left out without explanation. If you are not design-responsible, say so in the file. |  |  |  |
| 5 | Process flow diagram | Every step from receiving to shipping, including inspection, rework and scrap loops. | Shows the intended process rather than the line as it actually runs; rework loops missing. |  |  |  |
| 6 | Process FMEA | One row per step of the flow diagram, with actions for the high-risk failure modes. | Steps do not match the flow diagram; actions never reach the control plan. |  |  |  |
| 7 | Control plan | Every characteristic with its method, sample size and frequency, and a reaction plan for suspect product. | The reaction plan reads 'notify supervisor'; PFMEA actions are missing from it. |  |  |  |
| 8 | Measurement system analysis | Gage R&R and other studies for every gauge the control plan names. | The study was run on a different gauge from the one the control plan specifies. |  |  |  |
| 9 | Dimensional results | Every characteristic on the drawing measured against its tolerance, for each cavity, mould, die or line. | Balloon numbers do not match the drawing; one cavity measured on a multi-cavity tool. |  |  |  |
| 10 | Material and performance test results | Results against every material and performance requirement on the drawing and specifications. | A material certificate offered instead of results against your own specification. |  |  |  |
| 11 | Initial process studies | Capability (Ppk) for the special characteristics, from a run long enough to mean something. | Too few readings, or parts that were not consecutive. |  |  |  |
| 12 | Qualified laboratory documentation | The scope and accreditation of every laboratory that produced results in the file. | An external lab whose accreditation scope does not cover the test that was performed. |  |  |  |
| 13 | Appearance approval report | Colour, grain and texture evaluation for parts with appearance requirements. | Missing where appearance requirements exist, or submitted where they do not. |  |  |  |
| 14 | Sample production parts | Parts made on production tooling, with production gauges, operators and rate. | Parts from prototype tooling or a pre-production cell. |  |  |  |
| 15 | Master sample | A retained sample, identified and signed off, kept for as long as the part is active. | Not retained, or not identified as the master. |  |  |  |
| 16 | Checking aids | Every fixture and gauge used on the part, with calibration and MSA. | A fixture used for inspection that is neither in the MSA nor calibrated. |  |  |  |
| 17 | Customer-specific requirements | Evidence against every requirement in the customer's own supplier manual. | Treated as optional. Every customer adds its own. |  |  |  |
| 18 | Part submission warrant | The one-page summary, signed, with every field completed. | Signed before the rest of the file was complete; fields left blank. |  |  |  |

## The five submission levels — level 3 is the default

| Level | What goes to the customer | Typical use |
| --- | --- | --- |
| 1 | Part submission warrant only — plus the appearance approval report for appearance items. | Low-risk parts and repeat business, at the customer's discretion. |
| 2 | Warrant with product samples and limited supporting data. | Moderate risk; the customer states which data. |
| 3 | Warrant with product samples and complete supporting data. | The default level unless the customer specifies another. |
| 4 | Warrant and other requirements as defined by the customer. | Customer-defined combinations. |
| 5 | Warrant with product samples and complete supporting data, reviewed at your manufacturing location. | High-risk parts. The customer comes to you. |

## When a new PPAP is required

A new part, or a part new to this customer

Correction of a discrepancy on a previously submitted part

An engineering change to design records, specifications or materials

New, additional or modified tooling (perishable tools excepted)

Tooling or equipment transferred to a different plant or an additional location

A change of source for subcontracted parts, materials or services, such as heat treatment or plating

A change in the manufacturing process or method

Production from tooling inactive for volume production for twelve months or more

A change to a test or inspection method

## The numbers a reviewer checks

| What the reviewer checks | The number | Note |
| --- | --- | --- |
| Significant production run | 1 to 8 hours of production, at least 300 consecutive parts | Unless the customer specifies otherwise |
| Initial process study size | At least 25 subgroups and 100 readings | From consecutive parts of the significant run |
| Ppk — meets acceptance | Greater than 1.67 |  |
| Ppk — may be acceptable | 1.33 to 1.67 | Contact the customer before submitting |
| Ppk — does not meet | Below 1.33 | Expect to submit a corrective action plan |
| Gage R&R — acceptable | Below 10% | Of tolerance or process variation, per AIAG MSA |
| Gage R&R — may be acceptable | 10% to 30% | Depends on the application and the customer |
| Gage R&R — not acceptable | Above 30% |  |
| Record retention | As long as the part is active, plus one calendar year |  |

These are the AIAG PPAP and MSA defaults. Your customer's specific requirements override every one of them — read the customer manual before you plan the production run, not after.

Free PPAP checklist — 5xwhys.com/articles/rca/ppap/  ·  An element is done when its evidence is in the file, not when it is planned